Spritmesser zeigt falsch an./  2.7.2008

# How to check

Simple check.... connect the two wires on the petrol tank together, then switch ignition on, if the guage goes full then the sender in the tank is duff, if the guage still only reads half then it's either the gauge or voltage stabaliser, for it to be the stabaliser the temp gauge will shw the same symptoms as the fuel gauge.

## ERSATZTEIL

Cheers mate, jst checked the price for a new one on minispares, £17.63, mite take the current one out first and tinker with it to try and get it to work before i order a new one. Hopefully its jst a loose connection.

I've had this problem too for about 2 or 3 weeks now. I took the sender uni out the tank today and the float at the end of it had petrol in it but I couldn't see any visible holes. The float was about 1/4 full of petrol. So, I'll get a new sender unit. They're about £17 on minispares.com

Had the same problem with mine too up untill yesterday. Had to buy a new sender unit, more expensive than it looked too

I have the smae problem, I remember reading on another thread that it was usually caused by a break in the wire winding that forms the variable resistor in the sender. Once the float moves past the break there is no longer a circuit and the gauge reads empty. I also remember not being ablee to find an affordable replacement sender, so mine is staying as it is for the foreseable future.

My petrol gauge and temp gauge never move so am I reight in thinking this is time for a new voltage stabiliser??
Also do you think it may cure the problem of the indicators being permanently on, hazards flash though

Dave, I think it's more likely you have a duff connection somewhere or possibly a short circuit on the stabiliser supply. The indicator flasher needs the resistance of the bulbs to heat up its bimetallic strip, that's why they flash faster when a bulb is blown: it doubles the resistance. If it doesn't flash it's either worn out or there's not enough current going through it.

Stripes, quite possible you have mixed the wires up, if you have the supply from the gauge could be running straight to earth through the fuel tank. 

You can test a sender unit with a multimeter - the change in resistance should be linear as you move the float arm.

If it's just the float replace it with anything that floats and won't dissolve in petrol - pinch one off any scrapyard fuel tank.

Jammy did mine within like 10-15 mins, probably not even that. 

I helped watched, and it all made sense to me so it must be easy 

And now my gauge is fine I just have to learn to trust it again, thats the hard part Just make sure you are *very* low on petrol before you take the old one out!