#### Drive Shafts and Constant Velocity Joints

No.; Part No.; Description; Qty; Remarks. 1; 21A1760; R/H driveshaft rubber coupling type, CWE , 11/8 axle net; 1; 997/998 Cooper. 1; 21A1761; L/H driveshaft......................................CWE; 1; Minis pre discs. 1; 21A2295; R/H driveshaft, rubber coupling type, CWE; 1; 1275GT/'S'. 1; 21A2296; L/H driveshaft, rubber coupling type, CWE; 1; 1275GT/'S'. 2; GVC1104; R/H driveshaft less inboard pot joint was 22A1961; 1; Drum brakes 73 on. 2; GVC1103; L/H driveshaft less inboard pot joint was 22A1960; 1; Drum brakes 73 on. 2; GVC1100; R/H driveshaft less inboard pot joint, was 22A1963; 1; Disc except 'S'. 2; GVC1101; L/H driveshaft less inboard pot joint, was 22A1962; 1; Disc except 'S'. 3; 21A1852; R/H driveshaft, automatic; 1; CWE. 3; 21A1854; L/H driveshaft, automatic; 1; CWE. 3; 21A1857; R/H driveshaft Hardy Spicer type, CWE; 1; Cooper 'S'. 3; 21A1858; L/H driveshaft Hardy Spicer type, CWE; 1; Cooper 'S'. 4; 27H4775; R/H shaft only, except inboard pot joint type; 1; Upgraded material spec.. 4; 27H4776; L/H shaft only, except inboard pot joint type; 1; Upgraded material spec.. 4; MS1246; R/H shaft only, inboard pot joint only all 1973 on; 1; Upgraded material spec.. 4; MS1247; L/H shaft only, inboard pot joint only all 1973 on; 1; Upgraded material spec.. 5; GCV1105; Constant Velocity Joint was 17H8600; 2; 1 1/8 socket. 6; GCV1013; Outer constant velocity joint (Cooper S, late disc brakes); 2; 1 5/16 socket size. 7; GDG233; Budget Gaiter Kit for GCV1105. 7; 18G9027MS; Boot kit for either type CV joint (Does not foul hubs); 2; Aftermarket. 7; GSV1185; Boot kit for either type C.V.; 2; Unipart. 8; GSV1053; Boot kit for GCV1013 genuine; 2; Late type. 8; GDG202; Budget Boot kit for GCV1013. 9; AKF1457; Grease for CV joint; A/R; In gaiter kit. 10; 37H8706; Circlip inner, CV joint was 17H8596; 2; Extra or pot joint. 11; 17H8597; Circlip outer, CV joint thick; 2. 12; GCV1102; Inboard C.V. (pot) joint,; 2. 13; 37H8705; Oil seal for pot joint - to shaft; 2. 14; BHM7012; Gaiter kit for DAM667; 2. 14; GSV1073; Heavy duty gaiter for ADU4378 was BHM7205 (genuine); 2. 14; GDG234; Budget Boot kit for GCV11102. 15; 21A1696; Flange for rubber coupling type; 2. 16; 18G9029; Gaiter kit for 21A1696; 2. 17; GSV1192; Heavy duty Gaiter kit for 21A1696. 17; 21A963; Gaiter only; 2; As used on Hilo kit. 18; 27H7880; Hardy Spicer sliding joint; 2; Auto and 'S'. 19; GUJ101; Universal joint repair kit; 2; Auto and 'S.