SCHRAUBENGRÖSSEN BEIM MINI

# Bolt Stuff

Common sizes used on minis

| Front hub (drum) | 15/16" |
| --- | --- |
| Front hub (7" disk) | ? |
| Front hub (7.5" disk 'S') | 1 5/16" |
| Front hub (8.4" disk) | 1 5/16" |
| Rear hub | 15/16" |
| Steering wheel | 1 5/16" early, 1 1/8" later |
| Crank flywheel | 1 5/16" pre-verto clutch, 30mm verto ~1982> |
| Crank pulley | 1 1/4" or 1 5/16" |
| Cam sprocket | 1 1/4" |
| Crank pulley | 1 1/4" or 1 5/16" |
| Bumpers | Bumpers - Can be imperial or metric depending on source/age. Bumpers from Minispares are likely to be 1/4"UNF : Suggested by Andy as he bought some bumpers from Mini Spares and they came without nuts and he had quite a task trying to find replacement nuts not knowing the size! |
| OPRV cap | 1" |
| Hub swivel joint cap | 1.5" (deep socket) |
| Hub swivel-to-arm | 11/16" |
| Wheel nuts (imperial) | 11/16" |
| Wheel nuts (metric) | 17mm (usually) |
| Head nuts | 9/16" |
| Manifold nuts | 1/2" |
| Thermostat housing | 7/16" |
| Tappet cover bolts | 5/8" |
| Shock retaining nuts | 9/16" - this is same thread as wheel studs, yet different size socket |
| Sump plug | 15/16" (Tapered sump plugs 7/8") |
| Subframe Tower bolts (later single) | 1 5/16" inch (33.4mm) from Chris Morriss |
| Subframe Tower bolts ( earlky twin) | TBA |